Number  3               	February 14, 1998

Using Standard Automation’s InTime to Archive InSQL Server Data

# By Bryan Plaster

## Introduction

The following Tech Note discusses a procedure to automatically move the Industrial SQL Server historical data to the permanent location (preferably on another hard drive or other storage medium) for an online archiving solution.  This particular solution utilizes a scheduling utility that comes as part of Standard Automation’s Reporting Tools product offering.

Step 1

First you must configure the permanent storage location in Industrial SQL Server configure program (this is preferably on a different computer or hard drive than the circular location).

Step 2

Load up Standard Automation’s InTime and enter an appropriate action name

Click on the “Action” tab.

Fill it in with your directories from the Industrial SQL Server configure program.  “Source” would be your circular directory, and “Target” would be your permanent directory in InSQL Configure shown on the previous page.  This will move all of your files, except today’s history, to the permanent storage location for a true on-line archival system.

Click on the “Schedule” tab.

Configure a schedule for the archive operation to occur (i.e. on the first day of each month).  Click on “Accept” button and you are done.

Note:  Make sure that the InTime program is always running so that the scheduled operations will execute.