Number  30               	April 16, 1999

Using Standard Automation’s 16 bit Serial I/O Servers on Windows NT

# By Seann Gallagher

## Introduction

There are a number of I/O Servers that have not been, and may never be, converted into 32 bit applications.  16 bit serial servers, however, should have no problem communicating to their respective device on Windows NT.  The only issue that must be addressed is the recognition of the hardware key on the parallel port.  Because of the Windows NT HAL (Hardware Abstraction Layer), a device driver for NT must be loaded so that the server software can see the key.  The following will address what needs to be done.

### Obtaining the Hardlock Device Driver for Windows NT

Download HLDRV32.EXE from the Standard Automation FTP Server at ftp://ftp.stdauto.com/support/.

### Loading the Hardlock Device Driver for Windows NT

Execute the HLDRV32.EXE program.  You will see the following dialog.

Click on the Next button and the device driver files will be installed to your system.

Note:  You must have local administrative privileges on your computer so that the device can be added to the Windows NT registry.

You will be required to reboot the system so that the device driver is started.

### Verifying that the device driver is loaded and running

You can verify that device driver is loaded and running by looking in the Devices listing from Control Panel.  You will see the device “hardlock” in the list.  The Status will be “Started” if the device driver is running and the Startup will normally be “Automatic”.