Number  12	April 19, 1996

Adding TCP/IP for Windows 95 and Configuring WinSock for NetDDE  by Andrew Brodie

This Tech Note explains how to add the TCP/IP network protocol for Windows 95 and to configure Wonderware® NetDDE to communicate to another node using its WinSock interface.

# Procedure

- 1)	In order for NetDDE to communicate to another node using the WinSock interface, TCP/IP must be installed as a network protocol.  Double click the Network icon under the Control Panel, as shown below.  (Or from the Windows 95 desktop, right mouse click the Network Neighborhood icon and select Properties from the drop down menu.

- 2)	You should see the TCP/IP protocol that is listed under the Configuration tab on the Network dialog box, as shown below.  If you do not see TCP/IP listed, then click the Add button to add the TCP/IP protocol.

- 3)	If you added the TCP/IP protocol in step 2, follow this step.

- After you have added the TCP/IP protocol and verified that it is listed on the Network dialog box, double click the TCP/IP entry to configure its properties.  You should see the TCP/IP Properties dialog box, as shown below.

- Under the IP Address tab, enter the same IP Address that was assigned by your network administrator to that specific computer.  (For example, 111.111.222.222.)  Always enter a Subnet Mask of 255.255.255.0.

- 3)	Under the DNS Configuration tab (shown on the next page), enter the computer’s Host (or node node) and the Domain (or server) name.  (For example, HOST is entered for the Host and GENERIC.COM is entered for the Domain.)  Enter the IP Address for your Domain under DNS Server Search Order and click the Add button.

- 3)	Continued

- 4)	Under the Gateway tab, add the TCP/IP address of the node that was previously entered as the Host under the DNS Configuration tab.

- 5)	Once  the TCP/IP protocol has been successfully added, add the TCP/IP WinSock as a Network Interface by selecting Configure/Interfaces in the NetDDE menu.

The Tech Note publication is published periodically by the Wonderware Technical Support group.  Editor: Mari Fujii;  Technical Publications Coordinator:  Sabrina Haag;  Director of  Technical Support: Sheila S. Tyler;  Publisher, Wonderware Corporation, 100 Technology Drive, Irvine CA 92718.  E-mail your questions or requests to techpub@wonderware.com.

There is also technical information on the Wonderware software products on the Wonderware Bulletin Board Service at (714) 727-0726, on the Wonderware CompuServe® forum (GO WONDER), and on the Comprehensive Support Knowledge Base CD.  Call Wonderware Technical Support at (714) 727-3299 for more information on the BBS, the Wonderware forum, or the Knowledge Base CD.

Copyright 1996 Wonderware Corporation.  All Rights Reserved.  Wonderware is a registered trademark of the Wonderware Corporation in the United States of America and/or other countries.

April 19, 1995  Number 12