Number    38	February 4, 1997

Multiplexing InBatch Phases    by Sean Brennan

# Introduction

Many users of Wonderware’s Flexible Batch Management System, Wonderware® InBatch™ have come to realize that it is powerful and flexible when it comes to running multiple phases and batches and constructing flexible recipes.    However, one major concern is the number of connections required to model some processes.    A greater concern is the number of phases which must be programmed into the controller—one per connection—more if more than one phase is in the transfer class!    This Tech Note will describe a technique for multiplexing (mapping many to one or at least many to few) InBatch phases to controller phases based on a feature found in InBatch Release 4.1 and higher.

Two examples which can take advantage of this approach will be discussed.    The first involves a large tank farm for Bulk ingredients and a common header line to the processing units.    The second involves mobile vessels used for minor ingredient preparation.    Both of these examples, as well as other scenarios can use this approach.

# New Feature Introduced in InBatch 4.1

The feature introduced in InBatch 4.1 which allows this multiplexing of phases is the ability to assign multiple InBatch tags with the same address in your controller.    Now multiple InBatch tags can share the same memory in the PLC.    Phase multiplexing involves the assigning of multiple phase control, status, and parameter tags to the same addresses in the controller.    Note though that this method can only be used for phases which will never run at the same time.    If this were to happen, the second phase to run (using the same memory in the controller as the first phase) will encounter an unexpected phase status and InBatch will issue an error message.    So, part of our job is to construct our model in such a way as to insure that this does not happen.    Segments to the rescue!

Segments are used by InBatch to prevent multiple transfers from occurring at the same time.    Both of our examples will use segments.    The two examples shown here would probably have employed segments anyway since the physical construction of the process prevents multiple transfers from running simultaneously anyway.    This is exactly the situation in which phase multiplexing can be employed.

The best thing about phase multiplexing is that it gives the control system full InBatch functionality with minimal memory and little programming required in the controller.    Therefore, the benefits of Phase Multiplexing are:

1)	Reduces the memory burden on the controller when implementing a Comprehensive model in InBatch.

- Takes full benefit of the Comprehensive model (one which uses connections) which is the way InBatch was designed to work!

# Example 1

This example is a tank farm which consists of 20 bulk ingredient tanks (Process Class: BULK), connected by a common header line to our processing facility which includes ten processing vessels (Process Class: REACTOR)

Common Header HeaderHeader

BULK (20)

REACTOR (10)

Looking at the math, we see that 10 processing vessels multiplied by 20 bulk ingredient tanks equals 200 connections for the transfer class from BULK to REACTOR.    Our model will have a transfer phase on this class to move material from a BULK unit to a REACTOR unit.    This is a normal InBatch Comprehensive model which would normally require 200 phases to be written in the controller.    Each of these phases may require eleven discrete tags for control/status information and at least two analog tags for target and actual parameter information.    Add to that the logic required to control phase transitions and the control of processing equipment.    Multiply this by 200 phases and you can quickly see the burden on the controller.

It is a shame for this much controller memory and logic to be used for multiple transfers when only one of the transfers can run at any one time anyway!    (Since there is a common header, only one transfer should be run at a time.)    This is where Phase Multiplexing comes into play.

With Phase Multiplexing, we will model all of these connections (that is, a normal, comprehensive model) and we will program one phase in the controller.    This one phase will have 11 bits for control/status and two analog registers for target and actual data.    We will add a third register which will provide a source/destination code so that the controller phase knows which valves to open, which pumps to run, and other operations in order to complete the transfer.    The source/destination code will be unique to each connection in the transfer class.    We will model a segment common to all the connections to ensure that only one can be allocated at once.

Using TagLinker we will then assign all of our InBatch phase START tags to the address of our start command in the controller, all of the InBatch READY tags to the ready state bit in our controller, and so on.    The target and actual parameter extension tags in InBatch will be assigned to the controller target and actual registers.    Finally, all of the InBatch phase PREACT parameters extension tags will be assigned to our third controller register - source/destination code.    Preact is a constant value assigned to each connection in the model.    This constant indicates how much material will “trickle” into the destination unit after the valves have closed so that this value can be used by controller logic as an early warning to shut the valves in advance to presumably hit the target value every time.    InBatch sends this constant through a tag to the controller every time the phase is run.    The problem with this approach is (as most Preact users know) the value does not remain constant.    Pipes get blocked over time so the effective inside diameter of the pipe decreases.    There are climate fluctuations: materials run faster in the summer when it is warm than they do in the cold winter.    Preacts are usually material dependent as well, so if you change the material in the source unit the Preact value must change.    For reasons such as these, most facilities using Preact values have self-correcting Preacts.    This is most commonly done by applying a fraction of the error encountered on each run to the Preact value for the next run.

Therefore, a true Preact value is something that is best left to the controller.    Many facilities have access to these values through their MMI system.    With that said, we will use the InBatch Preact tag for a source/destination code.    Remember that the InBatch Preact is a constant value that is sent to the phase, the value of which comes directly from the connection that the phase is running on.    This is perfect since each connection can be assigned a unique “Preact” to identify itself to the multiplexed phase in the controller!    Now the controller will know which connection is triggering the transfer which means it knows exactly which valves and pumps to operate—remember there is a unique connection for every possible source/destination combination (200 in our example).

The Preact tag value is an analog number.    It may be an integer or a floating point value.    One approach is to use a code that will easily identify the source and destination such as source/destination code = 
(100 * source_id) + destination_id.    Using a rule such as this will make it easy for you and your controller to identify the source and destination units involved in the transfer.

In the end, our controller phase has 14 addresses for interfacing to InBatch (11 control/status bits + 1 Target + 1 Actual + 1 Preact (Source/Destination Code).    In TagLinker, there are 2800 tags for this information all being multiplexed to 14 addresses in the controller.    Now that is economy!

# Example 2

## Mobile Tank Example

This example involves mobile tanks supplying our REACTORs with minor ingredients that have been premixed .    The mobile vessels are reusable totes and they are identified as units in our process model.    Let us say that we have 80 of these totes which can supply product to any of our ten reactors.    That is 800 connections in all.    This time, only one mobile tote may be connected to a mixer at a time (through a flexible hose), but several mixers may have totes attached.    The solution is to provide one controller phase for each REACTOR so that we are multiplexing 800 phases in InBatch into ten phases in the controller.    The number of phases that you will need in the controller is always dictated by how many phases can be active at one time.

In this example it is not necessary for our Preact (that is, source/destination code) to provide the destination.    The destination is known by the phase because each phase is dedicated to a REACTOR destination.    One phase will handle all transfers to REACTOR1, another will handle all transfers to REACTOR 2, and so on.    Therefore, we can use the Preact tags to identify the source unit which is the mobile vessel.    In this example, we use the Preact as a source code and all connections from the same source unit will have the same code.    This will allow our phases in the controller to know which source tank is providing the material.

Why is this important if there is no automated valving for the mobile vessel?    After all, it makes no difference to your controller what mobile tank is connected at the end of the hose.    (We are assuming that the mobile tank has a manual valve which is opened by an operator when the unit is connected up to the REACTOR.    An automatic valve on the reactor allows product transfer.)    The answer lies in verification.    Standard operating procedure in most facilities that use mobile vessels, requires an operator to verify that the correct mobile vessel is connected before he opens the valve.    Wouldn’t it be nice if your phase could do this?    The operator reads the vessel id from the mobile tank and enters it.    This number is compared to the Preact (Source_Code) tag of the phase.    An exact match means that the correct mobile tank is in position and the material transfer can begin.    Better yet, you can affix bar code labels to the mobile vessels and have the operator scan the bar code before the phase opens the valve!

All transfer phases on all connections with the same destination will have common addressing in the controller.    The phase in the controller will be responsible for all phases with its REACTOR as the destination.    This means that all transfer phases going to REACTOR1 will have the same address for their START tags, but this address will be different for those phases with a destination of REACTOR2.    REACTOR2 has its own phase for its transfers.    We will model a segment on each REACTOR which is common to all connections leading to the REACTOR to make sure that only one is active.

In this example we have multiplexed 11,200 InBatch tags into 140 controller addresses which, again, is a huge saving.

# Updating the Preact Displayed Values

Who wants a model with Preact values labeled when they actually mean something else?    No problem!    All text in the InBatch configuration and runtime applications can be altered by editing simple text files.    You will find the Preact definition for ModelEdit in the file direktor.defaults which is located in the directory \InBatch\BatchSrv\app-defaults.    Edit the file and change the line that reads:

*dkPreactPrompt.labelString:		Preact:

to read

*dkPreactPrompt.labelString:		Source/Dest:

You can use whatever text you want.    In the same directory, there is the file ModelEdit.    Edit the file and look for the line:

*EditParameters*PreactToggle.labelString:		Preact

Again, change the “Preact” text to whatever you want.    Reboot your computer and restart Environment Display in order for the changes to take effect in the direktor.defaults file.    Then, enter ModelEdit.    The old Preact fields will now display the text that you entered.    Also, the runtime applications such as Batch Display will now use the new text as well.

There are also message files using Preact that are used by runtime applications such as Batch Display and TIM (used for InTouch wizards).    These files are in the directory \InBatch\BatchSrv\messages\.english (or other language).    The file BatchMngr.msg and ModelEdit.msg must be edited.    (Note that ModelEdit.msg has two occurrences of “Preact”).    Once these files are updated, the files Direktor.cat and DkInt.cat must be created in the message directory using the mkmsgcat function in the \InBatch\BatchSrv\bin directory, as follows:

1)	Use a text editor to change the BatchMngr.msg and ModelEdit.msg files in the .english (or other language) directory.

2)	Use a Command Prompt tool to execute the mkmsgcat commands by entering these commands:

cd \InBatch\BatchSrv\messages\.english

..\..\bin\mkmsgcat DkInt.msg DkInt.cat

..\..\bin\mkmsgcat Direktor.msg Direktor.cat

3)	Copy the .cat files up to the messages directory.

# Conclusion

Phase Multiplexing allows an InBatch system to take full advantage of a comprehensive model while using minimal memory and logic in the controller.    This can be a huge time saver for development.    This method also promotes the re-use of controller code for easy maintenance and changes.

The Tech Note publication is published periodically by the Wonderware Technical Support group.    Editor: Mari Fujii;    Technical Publications Coordinator: Sabrina Haag;    Director of    Technical Support: Sheila S. Tyler;    Publisher, Wonderware Corporation, 100 Technology Drive, Irvine CA 92618.    E-mail your questions or requests to techpubs@wonderware.com.

There is also technical information on the Wonderware software products on the WonderTech Web site at http://wondertech.wonderware.com, the Wonderware Bulletin Board Service at (714) 727-0726,    the Wonderware CompuServe forum (GO WONDER), the Comprehensive Support Knowledge Base CD, and the WonderFax FaxBack® service at (714) 450-5050.    Call Wonderware Technical Support at (714) 727-3299 for more information on the WonderTech Web site, the BBS, the Wonderware CompuServe forum, the Comprehensive Support Knowledge Base CD or the WonderFax system.

Copyright 1997 Wonderware Corporation.    All Rights Reserved.    Wonderware is a registered trademark of the Wonderware Corporation in the United States of America and/or other countries.                                                                                        February 4, 1997    Number 38