Number  43	March 3, 1997

Restoring the RLL Function Block Palette in InControl
by Andrew Brodie

Under certain circumstances, the Wonderware® InControl™ menu option for the RLL Function Block Palette may not appear in the InControl Development Environment window.  When you select Function Block Palette from the View menu and you select an RLL program for editing, the Function Block Palette should appear, as shown below:

However, in some situations, the RLL Function Block Palette may not appear.  For example, if you exit and restart ICDEV, but you forget to restore the palette, then there is a chance that the palette is moved off the screen.  This Tech Note issue shows how you can restore the missing palette by editing the Windows NT Registry.

# Procedure

Follow these steps to restore the missing RLL Function Block Palette in the InControl Development Environment:

- Close the InControl Development Environment window, then select Run from the Windows NT Start menu.  The Run dialog box should appear:

- Start up the Windows NT Registry Editor by typing regedt32 and then click the OK button.

- 2)	In the Registry Editor window, double-click the [HKEY_CURRENT_USER on Local Machine] entry.  Then, scroll down the registry tree and select the Software folder and then the sub-folder Wonderware.  Look for the ICDEV entry and select it, as shown here:

- Delete the ICDEV entry by pressing the <Delete> key.  A warning dialog box should appear:

- Click the Yes button to continue.

- The ICDEV entry should now be missing from the Wonderware folder under the [HKEY_CURRENT_USER on Local Machine] registry, as shown here:

- 3)	Close the Windows NT Registry Editor by selecting Exit from the Registry menu and start up the InControl Development Environment.

The RLL Function Block Palette should now appear.  If, for some reason, the RLL Function Block Palette should dissapear again, just repeat and follow the steps in this procedures.

The Tech Note publication is published periodically by the Wonderware Technical Support group.  Editors: Mari Fujii and Sabrina Haag;  Technical Publications Coordinator: Sabrina Haag;  Director of  Technical Support: Sheila S. Kester;  Publisher, Wonderware Corporation, 100 Technology Drive, Irvine CA 92618.  E-mail your questions or requests to techpubs@wonderware.com.

There is also technical information on the Wonderware software products on the WonderTech Web site at http://wondertech.wonderware.com, the Wonderware Bulletin Board Service at (714) 727-0726,  the Wonderware CompuServe forum (GO WONDER), the Comprehensive Support Knowledge Base CD, and the WonderFax FaxBack® service at (714) 450-5050.  Call Wonderware Technical Support at (714) 727-3299 for more information on the WonderTech Web site, the BBS, the Wonderware CompuServe forum, the Comprehensive Support Knowledge Base CD or the WonderFax system.

Copyright 1997 Wonderware Corporation.  All Rights Reserved.  Wonderware is a registered trademark of the Wonderware Corporation in the United States of America and/or other countries.

March 3, 1997  Number 43