Number  66                          	 May 23, 1997

Configuring the Microsoft IPX/SPX Protocol with NetBIOS and Wonderware NetDDE  by Geoff Olsen

Note  This Tech Note offers suggestions based on Wonderware Technical Support trial-and-error.  Note that Wonderware does not guarantee that this configuration will work and also does not support this configuration.

This Tech Note describes how to configure the Microsoft IPX/SPX-compatible network protocol with NetBIOS and Wonderware® NetDDE™ under Windows for Workgroups and Windows 95.  This assumes that your network interface hardware is properly configured and that your configuration will only use the IPX/SPX-compatible protocol with NetBIOS.   Wonderware Technical Support has not tested any other operating systems, network configurations, or protocols for this configuration.

Note  Your computer may prompt you to reboot during this configuration; however, do not reboot until you are instructed by this Tech Note.

# Windows 95 Configuration

## Step 1 - Adding the IPX/SPX-compatible Network Protocol

To add the IPX/SPX-compatible network protocol under Windows 95, first double-click the Network icon in the Windows Control Panel.  Then, click the Configuration tab in the Networks dialog box.  If you do not see IPX/SPX-compatible Protocol listed under the network components that are installed, then click the Add button.  In the Select Network Component Type dialog box, select Protocol and then click the Add button.  Next, double-click Microsoft in the list of manufacturers, select IPX/SPX-compatible Protocol from the list of network protocols, and then click the Add button.  You should see that the IPX/SPX-compatible protocol has been added to the list of network components that are installed.

Step 2 - Adding the Client for Microsoft Networks

While you are still in the Configuration tab of the Networks dialog box, if Client for Microsoft Networks is not listed under the network components that are installed, click the Add button.  Then, in the Select Network Component Type dialog box, select Client and click the Add button.  Next, double-click Microsoft in the list of manufacturers, select Client for Microsoft Networks from the list of network protocols, and then click the Add button.  You should see that the Client for Microsoft Networks has been added to the list of network components that are installed.

Step 3 - Enabling NetBIOS

To enable NetBIOS, double-click IPX/SPX- compatible Protocol, listed in the Networks dialog box under the network components that are installed.  Then, in the IPX/SPX- compatible Protocol Properties dialog box, check the box I want to enable NetBIOS over IPX/SPX, as seen below:

Click OK in the Networks dialog box to finish the configuration.  You may be prompted to insert the Windows 95 or Novell Client installation disks into your computer to complete the configuration.  Click OK when you are prompted to reboot your computer.

# Windows for Workgroups Configuration

Step 1 - Adding the IPX/SPX Compatible Transport with NetBIOS

To add the IPX/SPX-compatible transport with NetBIOS under Windows for Workgroups, first double-click the Network icon in the Windows Program Manager and then double-click the Network Setup icon.   If IPX/SPX Compatible Transport with NetBIOS is not listed under Network Drivers, then click the Drivers button.  In the Network Drivers dialog box, click the Add Protocol button, select IPX/SPX Compatible Transport with NetBIOS, and then click OK.  Remove any other protocols that are listed under Network Drivers, such as NetBEUI and TCP/IP.

The Network Drivers dialog box should now look similar to this:

Step 2 - Adding the Microsoft Windows Network

While you are still in the Network Setup dialog box, see if Microsoft Windows Network is listed under the network settings.  If it is not listed, then click the Networks button, click the Install Microsoft Windows Network radio button, and then click OK.

Click OK in both the Networks dialog box and the Network Setup dialog box to finish the configuration.  If you are prompted to make changes to your system files, click OK.  You may be asked to insert the Windows for Workgroups or Novell Client installation disks into your computer to complete the configuration.  Click OK when you are prompted to reboot your computer.

# Configuring Wonderware NetDDE for use with NetBIOS

After installing the IPX/SPX-compatible protocol with NetBIOS in Windows for Workgroups or Windows 95, install Wonderware NetDDE for Windows, and then reboot your computer.  Next, double-click the NetDDE icon.  In the NetDDE program, select Interfaces from the Configure menu.  Click the Add button in the Network Interface Selection dialog box, select NetBIOS under the Available Interfaces list, and then click OK.  You have now selected NetBIOS as the interface for use with NetDDE.  See the NetDDE for Windows User’s Guide to complete the configuration.

The Tech Note publication is published periodically by the Wonderware Technical Support group.  Editors: Mari Fujii and Sabrina Haag;  Technical Publications Coordinator: Sabrina Haag;  Director of  Technical Support: Sheila S. Kester;  Publisher, Wonderware Corporation, 100 Technology Drive, Irvine CA 92618.  E-mail your questions or requests to techpubs@wonderware.com.

There is also technical information on the Wonderware software products on the WonderTech Web site at http://wondertech.wonderware.com, the Wonderware Bulletin Board Service at (714) 727-0726,  the Wonderware CompuServe forum (GO WONDER), the Comprehensive Support Knowledge Base CD, and the WonderFax FaxBack® service at (714) 450-5050.  Call Wonderware Technical Support at (714) 727-3299 for more information on the WonderTech Web site, the BBS, the Wonderware CompuServe forum, the Comprehensive Support Knowledge Base CD or the WonderFax system.

Copyright 1997 Wonderware Corporation.  All Rights Reserved.  Wonderware is a registered trademark of the Wonderware Corporation in the United States of America and/or other countries.                                                 May 23, 1997  Number 66